ACL nima?

ACL — Access Control List — tarmoq trafikini ruxsat berish yoki bloklash uchun ishlatiladigan qoidalar ro‘yxati.

Oddiy qilib:

ACL = tarmoqdagi "kim → qayerga → nima qila oladi?" nazorati.

Masalan:

PC-A ──┐

├── Switch ── Router

PC-B ──┘

Biz shunday qoida berishimiz mumkin:

PC-A → Server       ✅ ruxsat

PC-B → Server       ❌ blok

PC-B → Internet     ✅ ruxsat

Bu yerda router trafikni ACL orqali tekshiradi.

2. ACL qanday ishlaydi?

Router paketni ko‘rganda ACL qoidalarini yuqoridan pastga tekshiradi.

Masalan:

1. PC-A ni blokla

2. Qolgan barcha trafikni ruxsat ber

Agar paket 1-qoidaga mos kelsa → deny.

Agar mos kelmasa → keyingi qoidaga o'tadi.

Juda muhim qoida

ACL oxirida ko‘rinmaydigan:

deny ip any any

mavjud deb hisoblanadi.

Shuning uchun:

permit ...

qoidalarini yozishni unutib qo‘ysak, qolgan trafik bloklanishi mumkin.

3. ACL turlari

Hozir ikkita asosiy turini bilib olamiz:

Standard ACL

Asosan manba IPga qaraydi.

Source → ruxsat/blok

Masalan:

10.10.10.10 → deny

Extended ACL

Ancha aniq nazorat qiladi:

Source IP

Destination IP

Protocol

Port

Masalan:

10.10.10.10 → Web Server → HTTP

yoki:

10.10.10.10 → Server → TCP 443

Real enterprise tarmoqlarda Extended ACL juda muhim.

4. Bugungi birinchi lab

Hozir birdan katta enterprise ACL qilmaymiz.

Avval Standard ACL bilan mexanizmni tushunib olamiz.

Topologiya:

PC-A

10.10.10.10

│

│

SW

│

│

R1

│

│

Server

10.20.20.10

Bizning maqsad:

PC-A Serverga kira olmasin.

Ya'ni:

PC-A ─────→ Server

❌ BLOCK

5. ACL yaratish

R1 ga o'tamiz.

Router> enable

Router# configure terminal

Router(config)# access-list 10 deny host 10.10.10.10

Router(config)# access-list 10 permit any

Bu ikki qatorning ma'nosi:

access-list 10 deny host 10.10.10.10

→ 10.10.10.10 manzildan kelayotgan trafikni blokla.

access-list 10 permit any

→ qolgan barcha manzillarga ruxsat ber.

Bu yerda 10 — ACL raqami.

Lekin ACL hali ishlamaydi

Bu juda muhim.

ACL yaratishning o‘zi yetarli emas.

Uni interface'ga qo‘llash kerak.

Lekin hozir buni shoshmasdan qilamiz.

Avval ACL'ni ko‘ramiz:

Router# show access-lists

Hozir shu komandani bajar.

Router#show access-lists Standard IP access list 10 10 deny host 10.10.10.10 20 permit any

Router#show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0     unassigned      YES unset  up                    up

GigabitEthernet0/1     unassigned      YES unset  administratively down down

Vlan1                  unassigned      YES unset  administratively down down

Ha, endi hammasi aniq. 👍

Hozir R1 holati:

G0/0    unassigned    up/up

G0/1    unassigned    administratively down/down

Vlan1   unassigned    administratively down/down

Demak G0/0 fizik jihatdan ulangan, lekin IP manzili yo‘q.

Biz ACL labini quyidagi kichik topologiyada qilamiz:

PC-A ─── SW1 ─── R1 G0/0

|

G0/1

|

Server

1-qadam — R1 G0/0 ga IP beramiz

PC-A uchun:

10.10.10.10/24

Gateway: 10.10.10.1

Shuning uchun R1:

Router# configure terminal

Router(config)# interface GigabitEthernet0/0

Router(config-if)# ip address 10.10.10.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# end

Keyin tekshir:

Router# show ip interface brief

G0/0:

GigabitEthernet0/0    10.10.10.1    ...    up    up

bo‘lishi kerak.

G0/0 → 10.10.10.1/24 → up/up

Endi PC-A ni Switch orqali G0/0 ga ulaymiz.

2-qadam — PC-A konfiguratsiyasi

PC-A → Desktop → IP Configuration:

IP Address:      10.10.10.10

Subnet Mask:     255.255.255.0

Default Gateway: 10.10.10.1

Topologiya:

PC-A

10.10.10.10

│

│

SW1

│

│

R1 G0/0

10.10.10.1

3-qadam — avval ACL'siz ping

PC-A → Command Prompt:

ping 10.10.10.1

Ping muvaffaqiyatli bo‘lishi kerak.

Xatolik

ping 10.10.10.1

Pinging 10.10.10.1 with 32 bytes of data:

Reply from 10.10.10.1: Destination host unreachable.

Reply from 10.10.10.1: Destination host unreachable.

Reply from 10.10.10.1: Destination host unreachable.

Reply from 10.10.10.1: Destination host unreachable.

Ping statistics for 10.10.10.1:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss),

Ha, bu yerda ACL hali sabab emas — biz uni interface'ga hali qo‘ymadik. Demak avval oddiy LAN ulanishini tuzatamiz.

Destination host unreachable chiqayotgan bo‘lsa, PC → Switch → R1 yo‘lida muammo bor.

1-qadam — PC IP sozlamasini tekshiramiz

PC-A → Desktop → IP Configuration ga kirib, quyidagilar aynan shunday ekanini tekshir:

IP Address:       10.10.10.10

Subnet Mask:      255.255.255.0

Default Gateway:  10.10.10.1

Keyin PC'da:

ipconfig

chiqishini ham ko‘r.

2-qadam — R1 da tekshiramiz

R1:

Router# show ip interface brief

Biz G0/0 uchun:

GigabitEthernet0/0    10.10.10.1    up    up

ekanini allaqachon ko‘rdik.

3-qadam — Switch portini tekshiramiz

SW1 da:

Switch# show interfaces status

Muammo

Router#show ip interface gigabitEthernet0/0

GigabitEthernet0/0 is up, line protocol is up (connected)

Internet address is 10.10.10.1/24

Broadcast address is 255.255.255.255

Address determined by setup command

MTU is 1500 bytes

Helper address is not set

Directed broadcast forwarding is disabled

Outgoing access list is not set

Inbound  access list is 10

Proxy ARP is enabled

Security level is default

Split horizon is enabled

ICMP redirects are always sent

ICMP unreachables are always sent

ICMP mask replies are never sent

IP fast switching is disabled

IP fast switching on the same interface is disabled

IP Flow switching is disabled

IP Fast switching turbo vector

IP multicast fast switching is disabled

IP multicast distributed fast switching is disabled

Router Discovery is disabled

Bu qatorda:

Inbound access list is 10

ACL G0/0 ga in yo‘nalishda qo‘yilgan.

Bizning ACL esa:

10 deny host 10.10.10.10

20 permit any

Shuning uchun PC-A (10.10.10.10) R1'ga paket yuborganda:

PC-A 10.10.10.10

│

▼

R1 G0/0

│

├── ACL 10

│

└── deny host 10.10.10.10 ❌

Natijada ping bloklanyapti. Demak ACL aynan o‘z vazifasini bajaryapti. 🎯

Endi ACL'ni amalda test qilamiz

PC-A'da yana:

ping 10.10.10.1

qil.

Hozir 100% loss chiqishi kerak, chunki biz PC-A'ni ataylab bloklaganmiz.

Keyin R1'ga:

Router# show access-lists

qilamiz.

ACL laboratoriyasining asosiy qismini muvaffaqiyatli bajarding.

Mana bu:

10 deny host 10.10.10.10 (12 match(es))

juda muhim.

Bu degani ACL 10 qoidasi 12 ta paketni ushlab, bloklagan.

PC'da:

Lost = 100%

chiqishi ham aynan shuni tasdiqlaydi. ✅

Hozir nima o‘rganding?

Bizda:

PC-A

10.10.10.10

│

▼

R1 G0/0

│

│ ACL 10

▼

deny host 10.10.10.10

ACL paketni R1'ning G0/0 interface'iga kirayotgan paytda tekshirdi va PC-A'ni blokladi.

Lekin bitta muhim narsani hali ko‘rsatmadik

ACL'dagi:

permit any

qismi nima qilishini ham amalda ko‘rishimiz kerak.

Ya'ni boshqa bir qurilma, masalan:

PC-B = 10.10.10.20

bo‘lsa:

PC-B → R1

ruxsat olishi kerak, chunki u 10.10.10.10 emas.

Shu orqali:

deny faqat kerakli hostni bloklaydi, permit any esa qolgan trafikni o'tkazadi

degan tushuncha amalda mustahkamlanadi.

Lekin hozircha PC-B qo‘shmaymiz. Avval Standard ACL'ning yana bir muhim xususiyati — wildcard maskni o‘rganamiz. Bu keyingi ACL'larda juda kerak bo‘ladi

bitta PC'ni bloklash → butun subnetni bloklash → faqat ma'lum tarmoqqa ruxsat berish

Endi Standard ACL + Wildcard Maskni tushunamiz. Bu ACL'larda juda muhim tushuncha.

2-qism — Wildcard Mask

Hozir biz:

deny host 10.10.10.10

deb bitta IPni blokladik.

Lekin tasavvur qil:

10.10.10.0/24

tarmog‘idagi barcha PC'larni bloklamoqchimiz.

Har bir IP'ni alohida yozish noqulay:

10.10.10.10

10.10.10.11

10.10.10.12

...

Buning o‘rniga wildcard mask ishlatamiz:

10.10.10.0 0.0.0.255

Wildcard maskni qanday tushunamiz?

Subnet mask:

255.255.255.0

Wildcard:

0.0.0.255

Ma'nosi:

- 0 → aynan shu qism mos kelishi kerak

- 255 → bu qism farq qilishi mumkin

Shuning uchun:

10.10.10.0 0.0.0.255

→ 10.10.10.x dagi barcha IP'larni bildiradi.

Masalan:

10.10.10.1    ✅

10.10.10.10   ✅

10.10.10.50   ✅

10.10.10.200  ✅

Kichik amaliy lab

Hozirgi ACL:

10 deny host 10.10.10.10

20 permit any

Biz uni o‘zgartirib:

10.10.10.0/24 tarmog‘idan kelayotgan barcha trafikni bloklash

qilamiz.

1-qadam

R1'da:

Router# configure terminal

Router(config)# no access-list 10

Router(config)# access-list 10 deny 10.10.10.0 0.0.0.255

Router(config)# access-list 10 permit any

Router(config)# end

Bu yerda:

deny 10.10.10.0 0.0.0.255

→ 10.10.10.0/24 tarmog‘idagi barcha manbalarni bloklaydi.

ACL hali ham G0/0'da qo‘llangan, shuning uchun qayta ip access-group qilish shart emas.

2-qadam — tekshirish

Router# show access-lists

Shunga o‘xshash chiqishi kerak:

Standard IP access list 10

10 deny 10.10.10.0 0.0.0.255

20 permit any

Natija kutilgandek bo’ldi

Wildcard mask bilan butun subnetni bitta ACL qoidasi orqali belgilashni tushunding.

Hozir ACL:

10 deny 10.10.10.0 0.0.0.255

20 permit any

degani:

10.10.10.0/24  → ❌ deny

boshqa tarmoqlar → ✅ permit

Endi eng muhim amaliy test

PC-A hali ham:

10.10.10.10

bo‘lgani uchun u ACL tomonidan bloklanishi kerak.

PC-A'da:

ping 10.10.10.1

qil.

Keyin R1'da:

show access-lists

qil.

deny 10.10.10.0 0.0.0.255 qatorida (match(es)) paydo bo‘lishi kerak.

Natija

Router#show access-lists

Standard IP access list 10

10 deny 10.10.10.0 0.0.0.255 (4 match(es))

20 permit any

C:\>ping 10.10.10.1

Pinging 10.10.10.1 with 32 bytes of data:

Reply from 10.10.10.1: Destination host unreachable.

Reply from 10.10.10.1: Destination host unreachable.

Reply from 10.10.10.1: Destination host unreachable.

Reply from 10.10.10.1: Destination host unreachable.

Ping statistics for 10.10.10.1:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss)

4 match(es) shuni ko‘rsatmoqda:

10 deny 10.10.10.0 0.0.0.255 (4 match(es))

PC'dan yuborilgan 4 ta ping paketining hammasi ACL tomonidan bloklangan.

Demak hozir sen ikki xil Standard ACL yozishni amalda ko‘rding:

deny host 10.10.10.10

→ faqat bitta host

va:

deny 10.10.10.0 0.0.0.255

→ butun /24 subnet

Keyingi bosqich — Extended ACL

Endi haqiqiy enterprise vaziyatga o'tamiz.

Masalan, kompaniyada:

Guest VLAN

│

▼

Server VLAN

Guest foydalanuvchilar:

- Web serverga HTTP/HTTPS orqali kira olsin ✅

- Lekin serverga SSH orqali kira olmasin ❌

- Boshqa kerakli trafik esa ruxsat etilsin.

Standard ACL bunday aniq nazoratni qila olmaydi, chunki u asosan source IPni tekshiradi.

Extended ACL esa:

Source IP

Destination IP

Protocol

Port

bo‘yicha qaror qila oladi.

Masalan:

Guest PC

10.10.10.10

│

│ TCP/80

▼

Web Server

10.20.20.10

✅

lekin:

Guest PC

10.10.10.10

│

│ TCP/22

▼

Server

10.20.20.10

❌

Shu — keyingi labimiz.

3-qism — Extended ACL

Standard ACL'da biz faqat manba IPni nazorat qildik.

Extended ACL esa ancha aniq ishlaydi:

Kim → qayerga → qaysi protokol → qaysi port orqali?

Masalan:

Guest PC

10.10.10.10

│

▼

Server

10.20.20.10

Biz quyidagicha qoida yaratishimiz mumkin:

10.10.10.10 → 10.20.20.10 → TCP/80  → permit

10.10.10.10 → 10.20.20.10 → TCP/22  → deny

Ya'ni Guest HTTP orqali serverga kira oladi, SSH orqali esa kira olmaydi.

1. Extended ACL sintaksisi

Umumiy ko‘rinishi:

access-list <raqam> <permit|deny> <protocol> <source> <destination> [port]

Masalan:

access-list 100 deny tcp host 10.10.10.10 host 10.20.20.10 eq 22

Buni qismlarga ajratamiz:

access-list 100

→ Extended ACL raqami.

deny

→ blokla.

tcp

→ TCP trafik.

host 10.10.10.10

→ aynan shu PC manba.

host 10.20.20.10

→ aynan shu server manzil.

eq 22

→ destination TCP port 22, ya'ni SSH.

Demak butun buyruq:

10.10.10.10 dan 10.20.20.10 ga SSH trafikni blokla.

2. Bizning lab

Hozir Standard ACL labimizdagi tarmoq:

PC-A

10.10.10.10

│

▼

SW1

│

▼

R1

│

▼

Server

10.20.20.10

Server tomonini ham R1 orqali ulaymiz.

R1:

G0/0 = 10.10.10.1/24

G0/1 = 10.20.20.1/24

Server:

IP:      10.20.20.10

Mask:    255.255.255.0

Gateway: 10.20.20.1

3. Avval Standard ACL'ni olib tashlaymiz

Hozir eski ACL 10 G0/0'da turibdi va PC'ni bloklayapti.

R1:

Router# configure terminal

Router(config)# interface GigabitEthernet0/0

Router(config-if)# no ip access-group 10 in

Router(config-if)# exit

Router(config)# no access-list 10

Router(config)# end

Bu bilan:

- ACL 10 interface'dan olib tashlanadi.

- Eski Standard ACL o‘chiriladi.

- Oddiy trafik yana o'tishi mumkin bo‘ladi.

Tekshiruv

Router# show access-lists

ACL 10 chiqmasligi kerak.

Hozir faqat shu ishni qilamiz.

Standard ACL 10 to‘liq olib tashlandi.

Hozir:

Router# show access-lists

hech narsa chiqarmayapti — demak ACL yo‘q.

Endi Extended ACL labining Server tomonini quramiz.

2-qadam — R1 G0/1 ni sozlaymiz

R1:

Router# configure terminal

Router(config)# interface GigabitEthernet0/1

Router(config-if)# ip address 10.20.20.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# end

Keyin:

Router# show ip interface brief

Biz quyidagini kutamiz:

GigabitEthernet0/1    10.20.20.1    ...    up    up

3-qadam — Server

Server → Desktop → IP Configuration:

IP Address:       10.20.20.10

Subnet Mask:      255.255.255.0

Default Gateway:  10.20.20.1

Topologiya:

PC-A                     Server

10.10.10.10              10.20.20.10

│                         │

▼                         ▼

SW1                      SW2

│                         │

└────── R1 ───────────────┘

G0/0       G0/1

.1         .1

Hozircha ACL yozmaymiz.

R1 ning ikkala tomoni ham tayyor:

G0/0 → 10.10.10.1 → up/up

G0/1 → 10.20.20.1 → up/up

Endi ACL yozishdan oldin oddiy routingni tekshiramiz. Bu professional troubleshooting'dagi muhim qoida:

Avval oddiy trafik ishlashini tasdiqlaymiz → keyin ACL qo‘shamiz → keyin ACL ta'sirini test qilamiz.

3-qadam — Serverni sozlash

Server → Desktop → IP Configuration:

IP Address:       10.20.20.10

Subnet Mask:      255.255.255.0

Default Gateway:  10.20.20.1

Keyin Server'dan:

ping 10.20.20.1

qil.

So‘ng PC-A'dan Serverga:

ping 10.20.20.10

qil.

Ikkalasi ham ishlashi kerak.

PC-A → Server ping ham ishladi.

Bu bizga quyidagilar to‘g‘ri ekanini tasdiqlaydi:

PC-A: 10.10.10.10/24 ✅

R1 G0/0: 10.10.10.1/24 ✅

R1 G0/1: 10.20.20.1/24 ✅

Server: 10.20.20.10/24 ✅

Gateway: 10.20.20.1 ✅

Routing: ishlayapti ✅

Hozircha ACL trafikni bloklamayapti ✅

Endi Extended ACL'ni qilamiz

Maqsadimiz:

PC-A → Server SSH (TCP/22) = BLOCK

PC-A → Server boshqa trafik = ALLOW

Buning uchun ACL'ni PC-A kelayotgan R1 G0/0 interfeysiga inbound qo‘yamiz.

Hozircha faqat ACL'ni yaratamiz. R1'da:

Router# configure terminal

Router(config)# access-list 100 deny tcp host 10.10.10.10 host 10.20.20.10 eq 22

Router(config)# access-list 100 permit ip any any

Router(config)# end

Bu yerda:

deny tcp → TCP trafikni blokla

10.10.10.10 → manba, ya'ni PC-A

10.20.20.10 → manzil, ya'ni Server

eq 22 → faqat SSH porti

permit ip any any → qolgan barcha IP trafiklarga ruxsat ber

Router#show access-lists

Extended IP access list 100

10 deny tcp host 10.10.10.10 host 10.20.20.10 eq 22

20 permit ip any any

ACL to‘g‘ri yaratildi ✅

Hozirgi ACL:

10 deny tcp host 10.10.10.10 host 10.20.20.10 eq 22

20 permit ip any any

Ma'nosi:

- 10 — PC-A (10.10.10.10) → Server (10.20.20.10) SSH/22 trafikini bloklaydi.

- 20 — qolgan IP trafiklarning barchasiga ruxsat beradi.

- ACL hali interfeysga qo‘yilmagan, shuning uchun hozircha real trafikni bloklamayapti.

Keyingi qadam

Endi ACL'ni R1 G0/0 inbound ga qo‘yamiz, chunki PC-A shu interfeys orqali R1'ga kiradi:

Router# configure terminal

Router(config)# interface GigabitEthernet0/0

Router(config-if)# ip access-group 100 in

Router(config-if)# end

Keyin tekshiramiz:

Router# show ip interface GigabitEthernet0/0

Natijada:

Inbound access list is 100

degan qator chiqishi kerak.

Extended ACL 100 R1 G0/0 inbound ga muvaffaqiyatli qo‘yildi.

Endi eng muhim qism — ACL haqiqatan ishlayotganini test qilamiz.

1️⃣ Avval Ping'ni tekshiramiz

PC-A'da:

ping 10.20.20.10

Bu ishlashi kerak, chunki biz faqat TCP port 22 (SSH) ni blokladik. Ping esa ICMP ishlatadi va permit ip any any orqali o'tadi.

Demak:

PC-A → Server

ICMP (ping) → ALLOW ✅

TCP/22 (SSH) → BLOCK ❌

Lekin SSH bloklanganini hali tekshirmadik.

Keyingi qadam — SSH'ni sinaymiz

Avval Server'da SSH xizmatini yoqamiz.

Server → Services → SSH

SSH'ni On qiling va quyidagicha user yarating:

Username: admin

Password: cisco123

Keyin PC-A'da:

ssh -l admin 10.20.20.10

Agar ACL to‘g‘ri ishlayotgan bo‘lsa, SSH ulanishi amalga oshmasligi kerak. ❌

Shundan keyin R1'da:

show access-lists

qilsak, deny tcp ... eq 22 qatorida match count oshganini ko‘ramiz.

Avval SSH'ni yoqing va PC-A'dan SSH qilib ko‘ring.

Katta real tarmoqlarda qanday bo‘ladi?

SSH'ni router yoki switch'ning o‘zida yoqish — juda oddiy va real amaliyot. Masalan:

Admin PC

│

▼

Management VLAN

│

▼

Switch / Router

Administrator qurilmaga:

ssh admin@10.x.x.x

orqali masofadan ulanadi.

Bu Telnet'dan xavfsizroq, chunki SSH trafikni shifrlaydi.

Lekin katta korxonada bitta muhim qoida bor

SSH'ni hamma joydan ochib qo‘yilmaydi.

Masalan:

Management PC ──► Router

└─► Switch

ruxsat beriladi.

Lekin:

Guest VLAN ──X──► Router SSH

Guest VLAN ──X──► Switch SSH

bloklanadi.

Mana shu joyda ACL juda muhim.

Bizning hozirgi Extended ACL labimiz nimani o‘rgatyapti?

Aslida juda foydali real-world kombinatsiyani:

SSH + ACL

Ya'ni:

"Kim qurilmaga SSH orqali kira oladi, kim kira olmaydi?"

Masalan:

Admin PC 10.10.10.10 ──► R1 SSH 22  ✅

Guest PC 10.10.60.10 ──X──► R1 SSH 22  ❌

Bu katta tarmoqlarda ishlatiladigan mantiq.

Faqat bitta narsani o‘zgartirish kerak

Hozir bizda:

PC-A → Server

uchun SSH ACL yozdik. Serverimiz SSH'ni qo‘llamagani uchun bu testni to‘liq bajara olmaymiz.

Shuning uchun menimcha, labni buzib qayta qurish shart emas. Biz mavjud R1'ni SSH server qilib, Extended ACL'ni:

PC-A → R1 SSH

uchun qo‘llab, real SSH + ACL testini qilamiz.

Bu foydali, real amaliyotga mos va sizning hozirgi Enterprise Networking bosqichingiz uchun juda yaxshi mashq.

SSH + Extended ACL ni real korxona mantiqida qilamiz.

Hozirgi holatda R1 SSH server bo‘ladi.

1-qadam — R1'da SSH sozlash

R1'ga:

Router# configure terminal

Router(config)# hostname R1

R1(config)# ip domain-name company.local

R1(config)# username admin privilege 15 secret cisco123

R1(config)# crypto key generate rsa

RSA so‘raganda:

How many bits in the modulus [512]:

1024 yozing.

Keyin:

R1(config)# ip ssh version 2

R1(config)# line vty 0 4

R1(config-line)# login local

R1(config-line)# transport input ssh

R1(config-line)# exit

R1(config)# end

2-qadam — tekshirish

R1# show ip ssh

SSH Enabled - version 2.0 chiqishi kerak.

R1 SSH server sifatida tayyor.

Hozir bizda:

- SSH version 2.0 ✅

- admin foydalanuvchi ✅

- VTY faqat SSH qabul qiladi ✅

- R1 manzili: 10.10.10.1 ✅

- PC-A: 10.10.10.10 ✅

3-qadam — ACLni SSH uchun moslaymiz

Hozirgi ACL Server'ga qaragan:

deny tcp host 10.10.10.10 host 10.20.20.10 eq 22

Uni R1'ga SSH ulanishini nazorat qiladigan qilib o‘zgartiramiz.

Avval eski ACL'ni olib tashlaymiz:

R1# configure terminal

R1(config)# interface GigabitEthernet0/0

R1(config-if)# no ip access-group 100 in

R1(config-if)# exit

R1(config)# no access-list 100

Hozircha shu yerda to‘xtang.

Bu qadamda ACL'ni olib tashladik, chunki keyingi ACL'ni toza holatda yaratamiz.

Bajarganingizdan keyin:

R1(config)# end

show access-lists bo‘sh chiqishi — ACL 100 muvaffaqiyatli o‘chirilgan degani.

Endi yangi Extended ACL yaratamiz.

4-qadam — SSH'ni faqat PC-A uchun boshqarish

Maqsadimiz:

PC-A 10.10.10.10 → R1 10.10.10.1 → SSH/22 ruxsat
Boshqa trafik → ruxsat

Avval ACL'ni yaratamiz:

R1# configure terminal

R1(config)# access-list 100 permit tcp host 10.10.10.10 host 10.10.10.1 eq 22

R1(config)# access-list 100 permit ip any any

R1(config)# end

Bu yerda:

- permit tcp → TCP trafik

- 10.10.10.10 → faqat PC-A

- 10.10.10.1 → R1

- eq 22 → faqat SSH

- permit ip any any → qolgan trafikni bloklab qo‘ymaslik uchun

Keyin:

ACL to‘g‘ri yaratildi.

Hozirgi qoida:

10 permit tcp host 10.10.10.10 host 10.10.10.1 eq 22

20 permit ip any any

Ya'ni PC-A'ga R1'ga SSH qilishga ruxsat berilgan.

Lekin bitta muhim nuqta bor: biz ACL'ni G0/0 inbound qo‘ysak, PC-A'dan R1'ga kelayotgan SSH paketini ACL tekshiradi. Shuning uchun bu yerda aynan shu joyni sinaymiz.

5-qadam — ACL'ni G0/0 ga qo‘yamiz

R1# configure terminal

R1(config)# interface GigabitEthernet0/0

R1(config-if)# ip access-group 100 in

R1(config-if)# end

Keyin tekshiring:

R1# show ip interface GigabitEthernet0/0

Inbound access list is 100 chiqishi kerak.

6-qadam — PC-A'dan R1'ga SSH

PC-A → Desktop → Command Prompt oching va:

ssh -l admin 10.10.10.1

Password so‘rasa:

cisco123

Agar hammasi to‘g‘ri bo‘lsa, PC-A R1'ga SSH orqali ulanadi va:

R1>

yoki

R1#

ko‘rinishidagi prompt olasiz. ✅

SH + Extended ACL testi muvaffaqiyatli ishladi.

Hozir biz amalda quyidagini isbotladik:

PC-A 10.10.10.10

│

│ SSH / TCP 22

▼

R1 10.10.10.1

│

▼

ACL 100 → permit ✅

Va R1'ga SSH orqali kirdingiz. Bu real tarmoqlarda ishlatiladigan boshqaruv usuli.

Endi ACL'ning ikkinchi tomonini tekshiramiz

Hozir ACL ruxsat beruvchi holatda:

10 permit tcp host 10.10.10.10 host 10.10.10.1 eq 22

20 permit ip any any

Endi aynan shu ACL orqali SSH'ni bloklashni ko‘ramiz.

Buning uchun 10-qatorni olib tashlab, uning o‘rniga deny qo‘yamiz.

R1'da:

R1# configure terminal

R1(config)# no access-list 100 permit tcp host 10.10.10.10 host 10.10.10.1 eq 22

R1(config)# access-list 100 deny tcp host 10.10.10.10 host 10.10.10.1 eq 22

R1(config)# end

Keyin:

R1# show access-lists

Natijada:

10 deny tcp host 10.10.10.10 host 10.10.10.1 eq 22

20 permit ip any any

bo‘lishi kerak.

SSH + Extended ACL testi muvaffaqiyatli ishladi.

Hozir biz amalda quyidagini isbotladik:

PC-A 10.10.10.10

│

│ SSH / TCP 22

▼

R1 10.10.10.1

│

▼

ACL 100 → permit ✅

Va R1'ga SSH orqali kirdingiz. Bu real tarmoqlarda ishlatiladigan boshqaruv usuli.

Endi ACL'ning ikkinchi tomonini tekshiramiz

Hozir ACL ruxsat beruvchi holatda:

10 permit tcp host 10.10.10.10 host 10.10.10.1 eq 22

20 permit ip any any

Endi aynan shu ACL orqali SSH'ni bloklashni ko‘ramiz.

Buning uchun 10-qatorni olib tashlab, uning o‘rniga deny qo‘yamiz.

R1'da:

R1# configure terminal

R1(config)# no access-list 100 permit tcp host 10.10.10.10 host 10.10.10.1 eq 22

R1(config)# access-list 100 deny tcp host 10.10.10.10 host 10.10.10.1 eq 22

R1(config)# end

Keyin:

R1# show access-lists

Natijada:

10 deny tcp host 10.10.10.10 host 10.10.10.1 eq 22

20 permit ip any any

bo‘lishi kerak.

SSH'ni bloklashga tayyor.

Keyingi test

PC-A'da yangi Command Prompt ochib:

ssh -l admin 10.10.10.1

qiling.

Bu safar SSH ulanishi amalga oshmasligi kerak. ❌

Keyin R1'da:

show access-lists qiling.

Ajoyib — aynan kutganimizdek ishladi. ✅

Demak, Extended ACL haqiqatan ham SSH'ni blokladi:

PC-A ──SSH/TCP 22──X──► R1

ACL 100

Endi bitta yakuniy tekshiruv qilamiz: ping ishlashi kerak, chunki biz faqat TCP/22 ni blokladik.

PC-A'da:

ping 10.10.10.1

Extended ACL laboratoriyasining asosiy maqsadini amalda bajardik.

Bizda:

| Trafik | Natija | Sabab |
| --- | --- | --- |
| PC-A → R1 SSH (TCP/22) | ❌ Bloklandi | deny tcp ... eq 22 |
| PC-A → R1 Ping (ICMP) | ✅ Ishladi | permit ip any any |

ACL:

Extended IP access list 100

10 deny tcp host 10.10.10.10 host 10.10.10.1 eq 22

20 permit ip any any

Muhim narsani o‘rgandik

Extended ACL yordamida faqat IP manzilni emas, balki:

- source IP

- destination IP

- protocol (tcp, udp, icmp)

- port (22, 80, 443 va hokazo)

bo‘yicha trafikni boshqarish mumkin.

Masalan:

deny tcp ... eq 22

→ SSH

deny tcp ... eq 80

→ HTTP

deny tcp ... eq 443

→ HTTPS

Shuningdek, ACL oxiridagi implicit deny sababli kerakli trafikni permit qilish muhimligini ham ko‘rdik.

🎯 Standard ACL + Wildcard + Extended ACL + SSH/ACL amaliy testi — bajarildi.